| MODEL | SP-30048 | SP-300512 | Configurable |
| --- | --- | --- | --- |
| - MAIN INFO |  |  |  |
| Rated Voltage | 48V | 51.2V |  |
| Rated Capacity | 300Ah |  |  |
| Battery Power | 13.44kWh | 15.36kWh |  |
| Battery Type | LFP |  |  |
| Cycling Lifespan | >6000 Cycles |  |  |
| Max.Parallel Capacity | 15 units |  | √ |

| - CHARGE&DISCHARGE |  |  |  |
| --- | --- | --- | --- |
| Max.Charging Voltage | 54V | 57.6V | √ |
| Dischare Cut-off Voltage | 40.5V | 44.8V | √ |
| Max.Charging Current | 205A |  | √ |
| Peak Charging Current | 210(1S) |  | √ |
| Max.Discharging Current | 205 |  | √ |
| Peak Discharging Current | 210(1S) |  | √ |
| - GENERAL |  |  |  |
| Net Weight | 115kg | 120kg |  |
| Gross Weight | 135kg | 140kg |  |
| Product Size(L×W×H) | 700×500×270MM |  |  |
| Package Size(L×W×H) | 980×640×465MM |  |  |
| Communication | RS485/RS232/CAN/Wifi/Bluetooth |  | √ |
| Storage Condition | 6months@25℃ 3months@35℃ 1month@45℃ |  |  |
| Charging Temp.Range | 0℃ to 65℃（32°F to 149°F) |  |  |
| Discharging Temp.Range | -10℃ to 65℃（314°F to 149°F) |  |  |
| Cooling Method | Passive Cooling |  |  |
| - STANDARD |  |  |  |
| Transportation | UN38.3/MSDS/TDG |  |  |

Solara  Power

Use "core" to specialize "core".

Floor-Standing Energy  Storage Lithium

Battery